Tender text

Brand: Phoenix Professional Audio

Type: MX-3 Built-in mixer preamplifier

The MX-3 is a universally applicable flush-mounted / surface-mounted mono preamp with two microphone inputs and a LINE input (mixable), XLR, phantom power and LED display.

Universal application, easy operation and high override resistance characterize the MX-3 preamplifier.

The pre-amplifier MX-3 is equipped with high-quality XLR mounting sockets (Neutrik), switchable phantom power 18 V for condenser microphones, LOW / HIGH ON-OFF jumpers per mic input and separate sound control LF / HF for each of the three inputs and master section.

The output of the MX-3 is transformer balanced with select jumper for 0 dB or +6 dB signal setting and provides high noise immunity at +6 dB setting of the output, so that the signal feedback can be done by JY(ST)Y with 2x2x0.6 line. The device is suitable for flush mounting and surface mounting.

Output is transformer balanced (galvanic isolation).

Technical Details:

- Mic inputs: balanced 2 x Mic inputs (0,5 - 5 mV / 1 kOhm), XLR standard

- Line inputs: unbalanced 1 x line inputs 0 dB, 0.775 V / 10 kOhm, CINCH

- Distortion: less than 0,3 %

- Tone control: +/- 12 dB

- Frequency response: 40 Hz - 20.000 Hz

- Outputs: 1 x balanced 0 dB bzw. +6 dB

- Phantom power: 18 V