Passive 2-way line array with 30 Watt power. Especially suitable for sound transmission in acoustically difficult rooms.

K1-30 speaker system consisting of:

Passive 2-way line array in very stable, resonance-free aluminium housing, tooled with 6 x 2“-broadband chassis [LF/MF] and a neodymium, highly efficient cylindrical wave tweeter driver in [HF] frequency range. The system has been developed for music and speech transmission with acoustically difficult room conditions with long reverberation time and demanding room architecture.

The aluminium housing is equipped with a highly sound-permeable front grille made of powder-coated aluminium grille and equipped with a moisture- and dust repellent acoustic foam. Surface  RAL 9016. The speaker system has a very precise radiation characteristics of H-130 degrees x V- 35 degrees, the range with SPL/92dB (PINK NOISE) is 8 meters. The speaker system is equipped with an integrated 30 Watt 100V-audio transformer and adjustment selector switch for nominal power@:1/1,1/2,1/3. Threaded holes M6 in the base, surface and at the rear side are integrated for ball joint bracket and mounting bracket entry. Simulation data EASE are available.

Delivery, mounting, connection

Specifications:

Frequency response: ..........    185 Hz - 20.000 Hz@-3dB

Nominal power@1/1 ....................................  30 Watt sine

Nominal power@1/2 ....................................  15 Watt sine

Nominal power@1/3 ...................................  7.5 Watt sine

Impedance@1/1 ................................................  333 Ohm

Impedance@1/2 ................................................  666 Ohm

Impedance@1/3 ..............................................  1.33 kOhm

Assembly: .............6 x 2“ [LF/MF], 1 x 1“ [HF] Neodymium

Input voltage ….................................... ...................  100V

Range@SPL92dB[PINK NOISE] ….....................  8 meter

Range@SPL95dB[PINK NOISE] ….....................  5 meter

Colour: ..........................................................      RAL 9016

Dimensions: .................... 600 (H) x 67 (W) x 73 (D) [mm]

Weight:  …................................................................ 2.8 kg

Model: Phoenix Professional Audio

Type: K1-30