Passive 2-way Line Array with 30 Watt power. Especially suitable for the sound transmission in acoustically difficult rooms.

K4-30N speaker system consisting of:

Passive 2-way Line Array in very stable, resonance-free aluminum housing, assembled with 3 x 3.5“ broadband neodymium chassis [LF/MF] and a neodymium, highly efficient cylindrical wave tweeter driver in [HF] frequency range. The system has been developed for music- and voice transmission in acoustically difficult space conditions with long reverberation time and superior room architecture.

The aluminum housing is equipped with a high-grade sound-permeable front grille made of powder-coated aluminum grille and a moisture- and dust-repellent acoustic foam. Surface RAL 9016. The speaker system has sound pressure SPL(1W/1m)@93[db] and a very precise directivity of H-90 degrees x V-40 degrees, the range at SPL/92dB (PINK NOISE) is 5 meter. The speaker system is equipped with an integrated 30 Watt 100V audio transformer and adaption selector switch for nominal power@: 1/1,1/2,1/4.

Threaded holes M6 in floor, upper side are integrated for entry of mounting brackets. Simulation files EASE are available.

Deliver, mount, connect

Specifications:

Frequency response: ..........    130 Hz - 18.000 Hz@-3dB

Nominal power@1/1: ...................................  30 Watt sine

Nominal power@1/2: ...................................  15 Watt sine

Nominal power@1/4: ..................................  7.5 Watt sine

Impedance@1/1: ...............................................  333 Ohm

Impedance@1/2: ...............................................  666 Ohm

Impedance@1/4: ............................................  1.33 kOhm

Assembly: …..... 3 x 3.5“ [LF/MF], 1 x 1“ [HF] Neodymium

Audio input: ….........................................................  100V

Range@SPL92dB[PINK NOISE]: ........................  5 meter

Range@SPL95dB[PINK NOISE]: ........................  3 meter

Passive switch [LF/HF]: …...…...…………...... 12db@oct.

Separation frequency: …….…….……………...… 4.5 kHz

Color: ...........................................................      RAL 9016

Dimensions: .................. 450 (H) x 95 (W) x 113 (D) [mm]

Weight:  …................................................................ 1.2 kg

Brand: Phoenix Professional Audio GmbH

Type: K4-30N